Nokia BB Automation

Automation Framework Guidelines Robot Test Cases & NB API (Device) Test Cases Coding standards, best practices and sample patterns for test script development Version: 0.1 ¦ Status: Draft Authors: Puspendra Upadhyay Date: February 2026

# Document Control

This section tracks the revision history of this Automation Framework Guidelines document.

| Version | Published By | Changes | Date |
| --- | --- | --- | --- |
| 0.1 | Puspendra Upadhyay | Initial Release | 18-02-2026 |

# 1. Automation Framework Guidelines for Robot Test Cases

## 1.1 Python File Guidelines

- Import and use get_registered_executor from connection_manager to create executors for devices.

- Always take device name as an input parameter in test functions to fetch the correct executor.

- Create separate executors for each device (e.g., DUT, RPI).

- Use the protocol variable to execute protocol-specific commands when required.

- Implement proper logging to console and custom log files.

- Avoid hard-coded sleep; prefer conditional waits when possible.

- Use try-except blocks for robust error handling.

Sample: Creating Device Executors

from execution.executor_helper import get_registered_executor def sample_executor_usage(device_name): executor = get_registered_executor(device_name) result = executor.execute("uptime") return result.stdout

Sample: Protocol-Aware Command Execution

def run_command(executor, protocol): if protocol.upper() == "MQTT": return executor.execute("show mqtt status") else: return executor.execute("ifconfig")

Sample: Connectivity Verification — Executor + Logging + Error Handling

Use this combined function as the standard pattern for verifying device connectivity. It combines executor creation, step logging, result validation and structured error handling in one place:

def connectivity_verification(device_name, remote_ip): executor = get_registered_executor(device_name) try: log_message(f"[STEP] Connectivity validation — pinging {remote_ip} from {device_name}") result = executor.execute(f"ping -c 3 {remote_ip}") if result.failed or "0% packet loss" not in result.stdout: raise AssertionError( f"Connectivity check FAILED\n" f"Device : {device_name}\n" f"Target : {remote_ip}\n" f"stdout : {result.stdout}\n" f"stderr : {result.stderr}" ) log_message(f"[OK] Connectivity verified — {device_name} can reach {remote_ip}") return True except AssertionError: log_message(f"[ERROR] Connectivity check failed for {device_name} -> {remote_ip}") raise except Exception as e: log_message(f"[ERROR] Unexpected error: {type(e).__name__}: {str(e)}") raise AssertionError(f"Unexpected error: {type(e).__name__}: {str(e)}")

NOTE: Pattern This function replaces three separate samples (executor creation, logging, error handling) with a single reusable pattern. Use it as the baseline for any step that executes a command and needs to validate the result.

## 1.3 Robot Framework File Guidelines

- Import connection_manager.py for device connect and disconnect operations.

- Import Python keyword files that contain test logic.

- Use Suite Setup and Suite Teardown for connection lifecycle management.

- Declare device JSON and test JSON as suite-level variables with ${EMPTY} defaults.

- Validate that DEVICE_JSON and TEST_JSON are not empty at the start of Load Device Info.

- Keep Test Cases minimal and keyword-driven.

- Create dedicated keywords for loading JSON and environment setup.

- Do not implement test logic inside Robot files.

- Use protocol from device JSON to control test execution.

Sample: Settings and Imports

*** Settings *** Library ../../../resources/keywords/BB-TRF-WAN-001.py Library ../../../resources/keywords/execution/connection_manager.py Library OperatingSystem Library Collections Resource ../../../resources/keywords/common_keywords.robot Suite Setup Initialize Test Environment Suite Teardown Cleanup Test Environment

Sample: Variables Block

*** Variables *** ${DEVICE_JSON} ${EMPTY} ${TEST_JSON} ${EMPTY} ${LOG_FOLDER} ${CURDIR}/../../../logs ${CONNECTION_PROTOCOL} SSH ${DEVICE_ID} ${EMPTY} ${EXECUTION_ID} ${EMPTY}

Sample: Test Case Structure

*** Test Cases *** BB-TRF-WAN-001 WAN End-to-End Test [Tags] BB-TRF-WAN-001 Log Message To Custom File ==== Starting Test ==== Run Setup Run Verification Run Teardown Log Message To Custom File ==== Test Successful ====

Sample: Suite Setup — FOR Loop Device Connection

Initialize Test Environment Initialize Custom Log File Load Device Info FOR ${device} IN @{DEVICE_LIST} OPEN DEVICE CONNECTION ... ${device["name"]} ... ${device["protocol"]} ... ${device["ip"]} ... ${device["user"]} ... ${device["pass"]} ... ${device["device_id"]} ... ${device["execution_id"]} END Cleanup Test Environment CLOSE ALL DEVICE CONNECTIONS

Sample: JSON Loading Keyword

*** Keywords *** Load Device Info Run Keyword If '${DEVICE_JSON}' == '' or '${TEST_JSON}' == '' ... Fail Device or Test JSON not provided ${device_config}= Evaluate json.loads(r'''${DEVICE_JSON}''') json ${test_config}= Evaluate json.loads(r'''${TEST_JSON}''') json ${DUT}= Set Variable ${device_config["BB"]} ${protocol}= Set Variable ${DUT.get("connection_protocol", "SSH")} ${dut_device}= Create Dictionary ... name=DUT protocol=${protocol} ip=${DUT["ip"]} ... user=${DUT["user"]} pass=${DUT["password"]} ... device_id=${DUT.get("device_id", "")} ... execution_id=${DUT.get("execution_id", "")} @{DEVICE_LIST}= Create List ${dut_device} Set Suite Variable ${DEVICE_LIST} Set Suite Variable ${CONNECTION_PROTOCOL} ${protocol}

NOTE: Best Practices Always use .get('key', 'default') for optional JSON fields. Never put test logic inside *** Test Cases *** — use wrapper keywords. Export all variables needed by keywords using Set Suite Variable.

## 1.3 Calling Python Functions from Robot Test Cases

This section explains how Python keyword functions are exposed to Robot Framework and how to call them correctly from .robot files.

### How the Bridge Works

Every Python function decorated with @keyword becomes directly callable by its keyword name from any Robot file that imports the Python file as a Library. The decorator name string becomes the Robot keyword name:

# Python side — BB-TRF-WAN-001.py from robot.api.deco import keyword from execution.executor_helper import get_registered_executor @keyword('VERIFY CONNECTIVITY') def connectivity_verification(device_name, remote_ip): executor = get_registered_executor(device_name) ...

# Robot side — BB-TRF-WAN-001.robot *** Settings *** Library ../../../resources/keywords/BB-TRF-WAN-001.py *** Test Cases *** BB-TRF-WAN-001 WAN Test VERIFY CONNECTIVITY DUT 34.235.14.203

NOTE: Rule The string passed to @keyword(...) is the exact keyword name used in Robot. If no string is given, Robot derives the name from the function name automatically. Always use UPPERCASE keyword names for custom keywords to distinguish from built-ins.

### Passing Arguments from Robot to Python

Arguments are passed positionally using the ... continuation syntax. Each line after the keyword name is one argument in order:

# Python function signature @keyword('SET UP GRE TUNNEL') def setup_gre_tunnel(device_name, tunnel_name, underlay_iface, local_ip, remote_ip, overlay_ip): ...

# Robot call — arguments passed in the same order as the function signature Run GRE Setup SET UP GRE TUNNEL ... DUT ... ${DUT_GRE['tunnel_name']} ... ${DUT_GRE['underlay_iface']} ... ${DUT_GRE['local_ip']} ... ${DUT_GRE['remote_ip']} ... ${DUT_GRE['overlay_ip']}

NOTE: Tip Use wrapper keywords (e.g., Run GRE Setup) in the *** Keywords *** section to pass suite variables into Python keyword calls. This keeps *** Test Cases *** clean — it only calls the wrapper, not the Python keyword directly.

### Capturing Return Values from Python

If a Python keyword returns a value, capture it into a Robot variable using the ${var}= syntax:

# Python — returns a value @keyword('GET DEVICE STATUS') def get_device_status(device_name): executor = get_registered_executor(device_name) result = executor.execute('uptime') return result.stdout.strip()

# Robot — capture the return value *** Keywords *** Check Device Uptime ${uptime}= GET DEVICE STATUS DUT Log To Console DUT uptime: ${uptime} Should Not Be Empty ${uptime} msg=Uptime response was empty

### Full Example — Python + Robot Bridge

The following shows a complete end-to-end example: a Python keyword file with two keywords and the Robot file that imports and calls them. This is the standard pattern for all BB Automation test cases:

# ── BB-TRF-WAN-001.py ────────────────────────────────────────── from robot.api.deco import keyword from robot.libraries.BuiltIn import BuiltIn from execution.executor_helper import get_registered_executor from datetime import datetime import os, traceback # Setup log file LOG_FILE = os.path.join(os.path.dirname(__file__), '../../logs/test.log') def log_message(msg): ts = datetime.now().strftime('%Y-%m-%d %H:%M:%S') line = f'[{ts}] {msg}' BuiltIn().log_to_console(line) with open(LOG_FILE, 'a') as f: f.write(line + ' ') @keyword('Initialize Custom Log File') def initialize_custom_log_file(): log_message('[STEP] Log initialized') @keyword('VERIFY CONNECTIVITY') def connectivity_verification(device_name, remote_ip): executor = get_registered_executor(device_name) try: log_message(f'[STEP] Pinging {remote_ip} from {device_name}') result = executor.execute(f'ping -c 3 {remote_ip}') if result.failed or '0% packet loss' not in result.stdout: raise AssertionError(f'Ping FAILED to {remote_ip}') log_message(f'[OK] {device_name} can reach {remote_ip}') return True except AssertionError: log_message(f'[ERROR] Connectivity failed: {device_name} -> {remote_ip}') raise except Exception as e: raise AssertionError(f'Unexpected error: {traceback.format_exc()}') @keyword('RUN WAN PING TEST') def run_wan_ping_test(device_name, target_ip, protocol): executor = get_registered_executor(device_name) log_message(f'[STEP] WAN ping test — protocol: {protocol}') if protocol.upper() == 'MQTT': result = executor.execute('show wan status') else: result = executor.execute(f'ping -c 5 {target_ip}') if result.failed: raise AssertionError(f'WAN ping test failed on {device_name}') log_message('[OK] WAN ping test passed') return result.stdout

# ── BB-TRF-WAN-001.robot ──────────────────────────────────────── *** Settings *** Library ../../../resources/keywords/BB-TRF-WAN-001.py Library ../../../resources/keywords/execution/connection_manager.py Resource ../../../resources/keywords/common_keywords.robot Suite Setup Initialize Test Environment Suite Teardown Cleanup Test Environment *** Variables *** ${DEVICE_JSON} ${EMPTY} ${TEST_JSON} ${EMPTY} ${CONNECTION_PROTOCOL} SSH *** Test Cases *** BB-TRF-WAN-001 WAN End-to-End Test [Tags] BB-TRF-WAN-001 Log Message To Custom File ==== Starting WAN Test ==== Run Connectivity Check Run WAN Test Log Message To Custom File ==== WAN Test Successful ==== *** Keywords *** # Suite Setup Initialize Test Environment Initialize Custom Log File Load Device Info FOR ${device} IN @{DEVICE_LIST} OPEN DEVICE CONNECTION ... ${device['name']} ${device['protocol']} ... ${device['ip']} ${device['user']} ... ${device['pass']} ${device['device_id']} ... ${device['execution_id']} END # Suite Teardown Cleanup Test Environment CLOSE ALL DEVICE CONNECTIONS # Wrapper keywords — call Python keywords with suite variables Run Connectivity Check VERIFY CONNECTIVITY DUT ${wan_remote_ip} Run WAN Test ${result}= RUN WAN PING TEST ... DUT ... ${wan_remote_ip} ... ${CONNECTION_PROTOCOL} Log To Console Result: ${result}

NOTE: Key Points 1. Library import makes all @keyword functions available in the robot file. 2. Wrapper keywords in *** Keywords *** inject suite variables into Python calls. 3. *** Test Cases *** only calls wrapper keywords — never Python keywords directly. 4. Return values from Python are captured with ${var}= in Robot.

# 2. Automation Framework Guidelines for NB API (Device) Test Cases

## 2.1 Python File Guidelines

### Step 1: Logging

Use the log() function from common_helper.py for all output. Never use print():

def log(message, *args, level="INFO"): timestamp = datetime.now().strftime("%Y-%m-%d %H:%M:%S") if args: try: message = message % args except Exception: message = f"{message} {args}"

### Step 2: Running Commands

Use run_local_command() for all shell commands. Use ping_remote() for connectivity checks:

def run_local_command(command, check=True, capture_output=True, text=True, shell=True, allow_fail=False): log("Executing command: %s", command) try: result = subprocess.run(command, shell=shell, capture_output=capture_output, text=text, check=check) return (result.stdout.strip() if result.stdout else "", result.stderr.strip() if result.stderr else "", result.returncode) except subprocess.CalledProcessError as e: if not allow_fail: log("Command failed (RC=%d): %s", e.returncode, command, level="FAIL") raise return ("", e.stderr.strip() if e.stderr else "", e.returncode) def ping_remote(iface, remote_ip): log("Pinging %s via %s...", remote_ip, iface) stdout, stderr, rc = run_local_command( f"ping -I {iface} -c 4 {remote_ip}", allow_fail=True ) if rc != 0 or "100% packet loss" in stdout or "0 received" in stdout: log("Ping failed.", level="ERROR") return False log("Ping successful.") return True

### Step 3: Test Pass / Fail Handling

Use EXIT_SUCCESS and EXIT_FAILED constants with sys.exit():

import sys EXIT_SUCCESS = 0 EXIT_FAILED = 1 # Example: stability test start = time.time() while time.time() - start < TEST_DURATION: if not ping_remote(iface, remote_ip): log("TEST FAILED !!!!! - Ping failed during stability test.") sys.exit(EXIT_FAILED) time.sleep(30) log(f"TEST PASSED !!!!! - Stability test completed for {TEST_DURATION} seconds.") sys.exit(EXIT_SUCCESS)

NOTE: Important Always end every NB API test with sys.exit(EXIT_SUCCESS) or sys.exit(EXIT_FAILED).